# Punch Accelerometer

This is a feedback and training device to help a boxer optimize punching force, reaction time, workout intensity, and endurance.

# Minimum Viable Product:

- Detect, record, and display g-force of a punch from a target

- Build with Arduino 101 with built-in accelerometer and Bluetooth

# Software Features for MVP

- Pattern recognition to detect straights/jabs, hooks, and uppercuts

- Bluetooth communication with computer or phone

- Call out punch force instead of displaying

- Audible prompt for punches

  - Measure reaction time after audible prompt

# Ultimate build – more hardware required

- Load cell could calibrate directly measured force (lbs) with acceleration of the target (g-force). This accomplishes 2 things: (1) gives you a real-world force measurement for punches (2) allows you to normalize readings from different targets (e.g. 70 lbs. heavy bag and BOB)

- Utilize built-in IMU to measure g-force to 16g, ADXL377 to measure from 16g to 200g

- Multiple accelerometers wired to one Arduino could select sensor with the highest force and use data to localize the punch (head/body)

- Connect heartrate strap and pace punches to match target HR zone

- Acceleration sensor on hands could add a lot of useful data:

  - Hand speed

  - Inefficiency - cocking arm before punching

  - Break out reaction time vs. time to target into separate measurements

- Pressure sensors in feet could correlate weight transfer to punching power

- Red light / green light prompts to mix in blocking and attacking

  - Challenge: this won’t measure avoiding punches (slipping/bobbing), only blocking

  - Red + green at the same time could prompt counterpunch, which would require boxer to reach target faster than usual

- Take video with forward-facing camera and sync to data feed

# Problems and ideas

- How to baseline power measurement? Use a load cell to compare directly measured force to force calculated from g-force detection.

http://www.digikey.com/product-detail/en/te-connectivity-measurement-specialties/FC2311-0000-0500-L/MSP6953-ND/809399

- Note that this has an output of 100mV, but the Arduino wants analog inputs of 3.3V. I think one of these amplifiers should solve the problem:

http://www.robotshop.com/en/strain-gauge-load-cell-amplifier-shield-2ch.html?gclid=CNqAhNSPhc0CFZNhfgodKigMhQ

https://learn.sparkfun.com/tutorials/load-cell-amplifier-hx711-breakout-hookup-guide

https://learn.sparkfun.com/tutorials/getting-started-with-load-cells

- 200g accelerometer to measure impacts over 16g limit of Curie:

https://www.adafruit.com/products/1413?gclid=CISd382-_8wCFVNqfgodWAcAMg

Ideally, you could employ both accelerometers, using the Curie built-in for acceleration up to 16g, and the ADXL for bigger hits

- Interesting sensor for cheap calibration, as well as to give accuracy points for landing a punch on the button: https://www.sparkfun.com/products/8685

- Nicely documented project with a 250g sensor:

https://abieneman.wordpress.com/2010/04/04/punch-acceleration-sensor/

- A different approach, but no useful code or parts list: http://www.eetimes.com/document.asp?doc_id=1279578

- Hold all accelerometer values in a 4x4 array:

  - Axis:

    - [0] Total

    - [1] ax

    - [2] ay

    - [3] az

  - Memory:

    - [0] current_reading

    - [1] previous_reading

    - [2] previous_punch

    - [3] peak_hold

# Math

- Calculating energy from a pendulum:

http://www.endmemo.com/physics/spendulum.php

http://www.wikihow.com/Calculate-Kinetic-Energy

Maximum punch speed = 10m/s

Arm mass = 6 kg

Kinetic energy = 300J

Similar reference punch: 68lb weight dropped from ceiling on a 1m pendulum – or –

45 lb weight on a 1.5m pendulum –or-

34 lb weight on a 2.0m pendulum

# Related Products

- Puncho boxing glove (development prototype): http://www.industrial-design-germany.com/innovations/intelligent-boxing-glove-puncho.html

- StrikeTec – Really slick UI, but doubt this will ever ship: http://efdsports.com/

- Hysko – taking pre-orders for Sept 2016 ship date @ $129. 2017 price planned @ $219. Raised $120K from Y-Combinator and $80K from other investors. https://www.hykso.com/

# Development Notes:

Goal: Detect, record, and display g-force of a punch from the target

5/30

- Detection working

- Display working and displaying g-force on a 3 axes

- Peak hold feature working – might want to make this a function

- Wrote Accelerometer-calibrate function to calibrate for starting orientation

Issues:

- Need to be able to factor out 1g gravity

CureIMU.setAcceleratorOffset() might fix this, but I couldn’t get it to work as I expected: https://www.arduino.cc/en/Reference/CurieIMUsetAccelerometerOffset

6/1

- Ordered load cell, amplifier, 200g accelerometer

- Added straight/uppercut/hook detection, but I’m not sure it’s working right

6/2

- Acceleration calculation verified

| X | Y | Z | G-Calculated | G-Displayed | Punch Type |
| --- | --- | --- | --- | --- | --- |
| 0.19 | 1.21 | 9.55 | 9.63 | 9.63 | S |
| 5.29 | 0.21 | 3.84 | 6.54 | 6.54 | H |
| 0.21 | 7.99 | 2.58 | 8.40 | 8.39 | U |
| 1.33 | 2.9 | 8.54 | 9.12 | 9.11 | S |
| 0.76 | 6.47 | 4.97 | 8.19 | 8.17 | U |
| 0.79 | 2.85 | 9.72 | 10.16 | 10.16 | S |

- Punch type detection working!

6/3

- Get Bluetooth working with this heartrate monitor sketch: https://www.arduino.cc/en/Tutorial/Genuino101CurieBLEHeartRateMonitor

6/4

- Set up Github repo

- Integrate GitHub repo w/Slack channel

- Display accelerometer readings from BLE: http://www.forward.com.au/pfod/BLE/index.html

  - Using nRF Master Control Panel, I see this is broadcasting as “101 BLE”, and accelerometer values are updating under the “TX Characteristic”

6/5

- Break sketch into multiple files

- Report TotalAcceleration to Bluetooth HeartRate service (AccelerationBLE sketch)

Next steps

- Get serial port working

- Record a stream of raw data

- Test IMU calibration sketch: https://www.arduino.cc/en/Tutorial/Genuino101CurieIMUOrientationVisualiser

- Test this sketch to visualize orientation of punching bag in real time:

https://www.arduino.cc/en/Tutorial/Genuino101CurieIMUOrientationVisualiser

- Create a reference for generating kinetic energy: weight swinging from a pendulum to hit a load cell

- As a second test, place load cell on far side of bag and have it strike a hard surface. This should measure how much energy is lost to deformation/compression of punching bag